CERTIFICATE

This is to certify that Mr./Ms. ….... Hemil…Chovatiya............. with enrolment no. ..........200303108003................. has successfully completed his/her    laboratory experiments in the Computer Organization & Architecture Lab(203105254) from the department of ........Information Technology(4ITA1)….…......... during the academic year ........2021-2022.........

Date of Submission: .........................                               Staff In charge: ...........................

Head of Department: ...........................................

| Sr. No | Experiment Title | Page No |  | Date of Performance | Date of Assessment | Marks (out of 10) | Sign |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | From | To |  |  |  |  |
| 1 | Write the working of 8085 simulator GNUsim8085 and basic architecture of 8085 along with small introduction. |  |  |  |  |  |  |
| 2 | Study the complete instruction set of 8085 and write the instructions in the instruction set of 8085 along with examples. |  |  |  |  |  |  |
| 3 | Write an assembly language code in GNUsim8085 to implement Addition of two 8bit Numbers. |  |  |  |  |  |  |
| 4 | Write an assembly language code in GNUsim8085 to implement Addition of two 16 bit Numbers. |  |  |  |  |  |  |
| 5 | Write an assembly language code in GNUsim8085 to implement Multiplication of two 8bit Numbers. |  |  |  |  |  |  |
| 6 | Write an assembly language code in GNUsim8085 to implement Division of two 8bit Numbers. |  |  |  |  |  |  |
| 7 | Write an assembly language code in GNUsim8085 to add two 8 bit numbers stored in memory and also storing the carry. |  |  |  |  |  |  |
| 8 | Write an assembly language code in GNUsim8085 to store numbers in reverse order in memory location. |  |  |  |  |  |  |

# INDEX

## PRACTICAL 1

### Aim: Write the working of 8085 simulator GNUsim8085 and basic architecture of 8085 along with small introduction.

- 8085 Microprocessor Architecture

- The architecture of the 8085 microprocessor mainly includes the timing & control unit, Arithmetic and logic unit, decoder, instruction register, interrupt control, a register array, serial input/output control.

- The most important part of the microprocessor is the central processing unit.

- The 8085 Bus Structure

The 8-bit 8085 CPU (or MPU – Micro Processing Unit) communicates with the other units using a 16-bit address bus, an 8-bit data bus and a control bus.

  - Address Bus

    - Consists of 16 address lines: A0 – A15

    - Operates in unidirectional mode: The address bits are always sent from the MPU to peripheral devices, not reverse.

    - 16 address lines are capable of addressing a total of 216 = 65,536 (64k) memory locations.

    - Address locations: 0000 (hex) – FFFF (hex)

  - Data Bus

    - Consists of 8 data lines: D0 – D7

    - Operates in bidirectional mode: The data bits are sent from the MPU to peripheral devices, as well as from the peripheral devices to the MPU.

    - Data range: 00 (hex) – FF (hex)

  - Control Bus

    - Consists of various lines carrying the control signals such as read / write enable, flag bits.

- The 8085: CPU Internal Structure

- The internal architecture of the 8085 CPU is capable of performing the following operations: Store 8-bit data (Registers, Accumulator).

- Perform arithmetic and logic operations (ALU)

- Test for conditions (IF / THEN)

- Sequence the execution of instructions

- Store temporary data in RAM during execution

  - The 8085: Registers

    - Six general purpose 8-bit registers: B, C, D, E, H, L

    - They can also be combined as register pairs to perform 16-bit operations: BC, DE, HL

    - Registers are programmable (data load, move, etc.)

  - Accumulator

    - Single 8-bit register that is part of the ALU !

    - Used for arithmetic / logic operations – the result is always stored in the accumulator.

  - Flag Bits

    - Indicate the result of condition tests.

    - Carry, Zero, Sign, Parity, etc.

    - Conditional operations (IF / THEN) are executed based on the condition of these flag bits.

  - Program Counter (PC)

    - Contains the memory address (16 bits) of the instruction that will be executed in the next step.

  - Stack Pointer (SP)

    - Stack pointer is a special purpose 16-bit register in the Microprocessor, which holds the address of the top of the stack.

- Example: Memory Read Operation

- Example: Instruction Fetch Operation

  - All instructions (program steps) are stored in memory.

  - To run a program, the individual instructions must be read from the memory in sequence, and executed.

  - Program counter puts the 16-bit memory address of the instruction on the address bus

  - Control unit sends the Memory Read Enable signal to access the memory

  - The 8-bit instruction stored in memory is placed on the data bus and transferred to the instruction decoder

  - Instruction is decoded and executed

## PRACTICAL 2

### Aim: Study the complete instruction set of 8085 and write the instructions with the instruction set along with examples

#### Instruction Set of 8085

- An instruction is a binary pattern designed inside a microprocessor to perform a specific function.

- The entire group of instructions that a microprocessor supports is called Instruction Set.

- 8085 has 246 instructions.

- Each instruction is represented by an 8-bit binary value.

- These 8-bits of binary value are called Op-Code or Instruction Byte.

#### Classification of Instruction Set

- Data Transfer Instruction

- Arithmetic Instructions

- Logical Instructions

- Branching Instructions

- Control Instructions

#### Data Transfer Instructions

- These instructions move data between registers, or between memory and registers.

- These instructions copy data from source to destination.

- While copying, the contents of source are not modified.

#### Data Transfer Instructions

| Opcode | Operand | Description |
| --- | --- | --- |
| MOV | Rd, Rs Rd, M M, Rs | Copy from source to destination. |

- This instruction copies the contents of the source register into the destination register. The contents of the source register are not altered.

- If one of the operands is a memory location, its location is specified by the contents of the HL registers.

- Example: MOV B, C

- MOV B, M

- MOV M, C

| Opcode | Operand | Description |
| --- | --- | --- |
| MVI | Rd, Data M, Data | Move immediate 8-bit |

- The 8-bit data is stored in the destination register or memory.

- If the operand is a memory location, its location is specified by the contents of the H-L registers. Example: MVI A, 57H

- MVI M, 57H

| Opcode | Operand | Description |
| --- | --- | --- |
| LXI | Reg. pair, 16-bit data | Load register pair immediate |

- This instruction loads 16-bit data in the register pair.

- Example: LXI H, 2034 H

| Opcode | Operand | Description |
| --- | --- | --- |
| LDA | 16-bit address | Load Accumulator |

- The contents of a memory location, specified by a 16- bit address in the operand, are copied to the accumulator.

- The contents of the source are not altered.

- Example: LDA 2034H

| Opcode | Operand | Description |
| --- | --- | --- |
| LDAX | B/D Register Pair | Load accumulator indirect |

- The contents of the designated register pair point to a memory location.

- This instruction copies the contents of that memory location into the accumulator.

- The contents of either the register pair or the memory location are not altered.

- Example: LDAX B

| Opcode | Operand | Description |
| --- | --- | --- |
| LHLD | 16-bit address | Load H-L registers direct |

- This instruction copies the contents of memory location pointed out by 16-bit address into register L.

- It copies the contents of the next memory location into register H.

- Example: LHLD 2040 H

| Opcode | Operand | Description |
| --- | --- | --- |
| STA | 16-bit address | Store accumulator direct |

- The contents of the accumulator are copied into the memory location specified by the operand.

- Example: STA 2500 H

| Opcode | Operand | Description |
| --- | --- | --- |
| STAX | Reg. pair | Store accumulator indirect |

- The contents of the accumulator are copied into the memory location specified by the contents of the register pair.

- Example: STAX B

| Opcode | Operand | Description |
| --- | --- | --- |
| SHLD | 16-bit address | Store H-L registers direct |

- The contents of register L are stored into memory location specified by the 16-bit address.

- The contents of register H are stored into the next memory location.

- Example: SHLD 2550 H

| Opcode | Operand | Description |
| --- | --- | --- |
| XCHG | None | Exchange H-L with D-E |

- The contents of register H are exchanged with the contents of register D.

- The contents of register L are exchanged with the contents of register E.

- Example: XCHG

#### Arithmetic Instructions

These instructions perform the operations like:

- Addition

- Subtract

- Increment

- Decrement

| Opcode | Operand | Description |
| --- | --- | --- |
| ADD | R M | Add register or memory to accumulator |

- The contents of the register or memory are added to the contents of the accumulator.

- The result is stored in the accumulator.

- If the operand is memory location, its address is specified by H-L pair.

- All flags are modified to reflect the result of the addition.

- Example: ADD B or ADD M

| Opcode | Operand | Description |
| --- | --- | --- |
| ADC | R M | Add register or memory to accumulator with carry |

- The contents of register or memory and Carry Flag (CY) are added to the contents of the accumulator.

- The result is stored in the accumulator.

- If the operand is memory location, its address is specified by H-L pair.

- All flags are modified to reflect the result of the addition.

- Example: ADC B or ADC M

| Opcode | Operand | Description |
| --- | --- | --- |
| ADI | 8-bit data | Add immediate to accumulator |

- The 8-bit data is added to the contents of the accumulator.

- The result is stored in the accumulator.

- All flags are modified to reflect the result of the addition.

- Example: ADI 45 H

| Opcode | Operand | Description |
| --- | --- | --- |
| ACI | 8-bit data | Add immediate to accumulator with carry |

- The 8-bit data and the Carry Flag (CY) are added to the contents of the accumulator.

- The result is stored in the accumulator.

- All flags are modified to reflect the result of the addition.

- Example: ACI 45 H

| Opcode | Operand | Description |
| --- | --- | --- |
| DAD | Reg. pair | Add register pair to H-L pair |

- The 16-bit contents of the register pair are added to the contents of the H-L pair.

- The result is stored in H-L pair.

- If the result is larger than 16 bits, then CY is set.

- No other flags are changed.

- Example: DAD B

| Opcode | Operand | Description |
| --- | --- | --- |
| SUB | R M | Subtract register or memory from accumulator |

- The contents of the register or memory location are subtracted from the contents of the accumulator.

- The result is stored in the accumulator.

- If the operand is memory location, its address is specified by H-L pair.

- All flags are modified to reflect the result of subtraction.

- Example: SUB B or SUB M

| Opcode | Operand | Description |
| --- | --- | --- |
| SUI | 8-bit data | Subtract immediate from accumulator |

- The 8-bit data is subtracted from the contents of the

- accumulator.

- The result is stored in the accumulator.

- All flags are modified to reflect the result of subtraction.

- Example: SUI 45 H

| Opcode | Operand | Description |
| --- | --- | --- |
| SBI | 8-bit data | Subtract immediate from accumulator with borrow |

- The 8-bit data and the Borrow Flag (i.e. CY) is subtracted from the contents of the accumulator.

- The result is stored in the accumulator.

- All flags are modified to reflect the result of subtraction.

- Example: SBI 45 H

| Opcode | Operand | Description |
| --- | --- | --- |
| INR | R M | Increment register or memory by 1 |

- The contents of register or memory location are incremented by 1.

- The result is stored in the same place.

- If the operand is a memory location, its address is specified by the contents of H-L pair.

- Example: INR B or INR M

| Opcode | Operand | Description |
| --- | --- | --- |
| INX | R | Increment register pair by 1 |

- The contents of the register pair are incremented by 1.

- The result is stored in the same place.

- Example: INX H

| Opcode | Operand | Description |
| --- | --- | --- |
| DCR | R M | Decrement register or memory by 1 |

- The contents of register or memory location are decremented by 1.

- The result is stored in the same place.

- If the operand is a memory location, its address is specified by the contents of H-L pair.

- Example: DCR B or DCR M

#### Logical Instructions

- These instructions perform logical operations on data stored in registers, memory and status flags.

The logical operations are:

- AND

- OR

- XOR

- ROTATE

- COMPARE

- COMPLEMENT

| Opcode | Operand | Description |
| --- | --- | --- |
| CMP | R M | Compare register or memory with accumulator |

- The contents of the operand (register or memory) are compared with the contents of the accumulator.

- Both contents are preserved .

- The result of the comparison is shown by setting the flags of the PSW as follows:

- if (A) < (reg/mem): carry flag is set

- if (A) = (reg/mem): zero flag is set

- if (A) > (reg/mem): carry and zero flags are reset.

- Example: CMP B or CMP M

| Opcode | Operand | Description |
| --- | --- | --- |
| CPI | 8-bit data | Compare immediate with accumulator |

- The 8-bit data is compared with the contents of the accumulator.

- The values being compared remain unchanged.

- The result of the comparison is shown by setting the flags of the PSW as follows:

- if (A) < data: carry flag is set

- if (A) = data: zero flag is set

- if (A) > data: carry and zero flags are reset

- Example: CPI 89H

| Opcode | Operand | Description |
| --- | --- | --- |
| ANA | R M | Logical AND register or memory with accumulator |

- The contents of the accumulator are logically ANDed with the contents of register or memory.

- The result is placed in the accumulator.

- If the operand is a memory location, its address is specified by the contents of H-L pair.

- S, Z, P are modified to reflect the result of the operation.

- CY is reset and AC is set.

- Example: ANA B or ANA M.

| Opcode | Operand | Description |
| --- | --- | --- |
| ANI | 8-bit data | Logical AND immediate with accumulator |

- The contents of the accumulator are logically ANDed with the 8-bit data.

- The result is placed in the accumulator.

- S, Z, P are modified to reflect the result.

- CY is reset, AC is set.

- Example: ANI 86H.

| Opcode | Operand | Description |
| --- | --- | --- |
| XRA | R M | Exclusive OR register or memory with accumulator |

- The contents of the accumulator are XORed with the contents of the register or memory.

- The result is placed in the accumulator.

- If the operand is a memory location, its address is specified by the contents of H-L pair.

- S, Z, P are modified to reflect the result of the operation.

- CY and AC are reset.

- Example: XRA B or XRA M.

| Opcode | Operand | Description |
| --- | --- | --- |
| ORA | R M | Logical OR register or memory with accumulator |

- The contents of the accumulator are logically ORed with the contents of the register or

- memory.

- The result is placed in the accumulator.

- If the operand is a memory location, its address is specified by the contents of H-L pair.

- S, Z, P are modified to reflect the result, CY and AC are reset.

- Example: ORA B or ORA M.

| Opcode | Operand | Description |
| --- | --- | --- |
| ORI | 8-bit data | Logical OR immediate with accumulator |

- The contents of the accumulator are logically ORed with the 8-

- bit data.

- The result is placed in the accumulator.

- S, Z, P are modified to reflect the result.

- CY and AC are reset.

- Example: ORI 86H.

| Opcode | Operand | Description |
| --- | --- | --- |
| XRA | R M | Logical XOR register or memory with accumulator |

- The contents of the accumulator are XORed with the contents of the register or memory.

- The result is placed in the accumulator.

- If the operand is a memory location, its address is specified by the contents of H-L pair.

- S, Z, P are modified to reflect the result of the operation.

- CY and AC are reset.

- Example: XRA B or XRA M.

#### Branching Instructions

- The branching instruction alter the normal sequential flow.

- These instructions alter either unconditionally or conditionally.

| Opcode | Operand | Description |
| --- | --- | --- |
| JMP | 16-bit address | Jump unconditionally |

- The program sequence is transferred to the memory location specified by the 16-bit address given in the operand.

- Example: JMP 2034 H.

| Opcode | Operand | Description |
| --- | --- | --- |
| Jx | 16-bit address | Jump conditionally |

- The program sequence is transferred to the memory location specified by the 16-bit address given in the operand based on the specified flag of the PSW.

- Example: JZ 2034 H.

#### Jump Conditionally

| Opcode | Operand | Description |
| --- | --- | --- |
| JC | Jump if Carry | CY = 1 |
| JNC | Jump if No Carry | CY = 0 |
| JP | Jump if Positive | S = 0 |
| JM | Jump if Minus | S = 1 |
| JZ | Jump if Zero | Z = 1 |
| JNZ | Jump if No Zero | Z = 0 |
| JPE | Jump if Parity Even | P = 1 |
| JPO | Jump if Parity Odd | P = 0 |

| Opcode | Operand | Description |
| --- | --- | --- |
| CALL | 16-bit address | Call unconditionally |

- The program sequence is transferred to the memory location specified by the 16-bit address given in the operand.

- Before the transfer, the address of the next instruction after CALL (the contents of the program counter) is pushed onto the stack.

- Example: CALL 2034 H.

| Opcode | Operand | Description |
| --- | --- | --- |
| Cx | 16-bit address | Call unconditionally |

- The program sequence is transferred to the memory location specified by the 16-bit address given in the operand based on the specified flag of the PSW.

- Before the transfer, the address of the next instruction after the call (the contents of the program counter) is pushed onto the stack.

- Example: CZ 2034 H.

Call Conditionally

| Opcode | Operand | Description |
| --- | --- | --- |
| CC | Call if Carry | CY = 1 |
| CNC | Call if No Carry | CY = 0 |
| CP | Call if Positive | S = 0 |
| CM | Call if Minus | S = 1 |
| CZ | Call if Zero | Z = 1 |
| CNZ | Call if No Zero | Z = 0 |
| CPE | Call if Parity Even | P =1 |
| CPO | Call if Parity Odd | P = 0 |
| Opcode | Operand | Description |
| RET | None | Return unconditionally |

- The program sequence is transferred from the subroutine to the calling program.

- The two bytes from the top of the stack are copied into the program counter, and program execution begins at the new address.

- Example: RET.

| Opcode | Operand | Description |
| --- | --- | --- |
| Rx | None | Call conditionally |

- The program sequence is transferred from the subroutine to the calling program based on the specified flag of the PSW.

- The two bytes from the top of the stack are copied into the program counter, and program execution begins at the new address.

- Example: RZ.

#### Return Conditionally

| Opcode | Operand | Description |
| --- | --- | --- |
| RC | Return if Carry | CY = 1 |
| RNC | Return if No Carry | CY = 0 |
| RP | Return if Positive | S = 0 |
| RM | Return if Minus | S = 1 |
| RZ | Return if Zero | Z = 1 |
| RNZ | Return if No Zero | Z = 0 |
| RPE | Return if Parity Even | P =1 |
| RPO | Return if Parity Odd | P = 0 |

| Opcode | Operand | Description |
| --- | --- | --- |
| RST | 0-7 | Restart (Software Interrupts) |

- The RST instruction jumps the control to one of eight memory locations depending upon the number.

- These are used as software instructions in a program to transfer program execution to one of the eight locations.

- Example: RST 3.

#### Restart Address Table

| Instructions | Restart Address |
| --- | --- |
| RST 0 | 0000 H |
| RST 1 | 0008 H |
| RST 2 | 0010 H |
| RST 3 | 0018 H |
| RST 4 | 0020 H |
| RST 5 | 0028 H |
| RST 6 | 0030 H |
| RST 7 | 0038 H |

Control Instructions

| Opcode | Operand | Description |
| --- | --- | --- |
| NOP | None | No operation |

- No operation is performed.

- The instruction is fetched and decoded but no operation is executed.

- Example: NOP

| Opcode | Operand | Description |
| --- | --- | --- |
| HLT | None | Halt |

- The CPU finishes executing the current instruction and halts any further execution.

- An interrupt or reset is necessary to exit from the halt state.

- Example: HLT

| Opcode | Operand | Description |
| --- | --- | --- |
| DI | None | Disable interrupt |

- The interrupt enable flip-flop is reset and all the interrupts except the TRAP are disabled.

- No flags are affected.

- Example: DI

| Opcode | Operand | Description |
| --- | --- | --- |
| EI | None | Enable interrupt |

- The interrupt enable flip-flop is set and all interrupts are enabled.

- No flags are affected.

- This instruction is necessary to re-enable the interrupts (except TRAP).

- Example: EI

| Opcode | Operand | Description |
| --- | --- | --- |
| RIM | None | Read interrupt mask |

- This is a multipurpose instruction used to read the status of interrupts 7.5, 6.5, 5.5 and read serial data input bit.

- The instruction loads eight bits in the accumulator with the following interpretations.

- Example: RIM

## PRACTICAL 3

### AIM: Write an assembly language code in GNUsim8085 to implement Addition of two  8 bit Numbers.

Theory:

| Code | Meaning |
| --- | --- |
| LDA 0001 | Load value of memory location 0001 in Accumulator A |
| MOV B,A | Move data from memory to accumulator |
| STA 0003 | Store accumulator contents in memory |
| ADD B | Add data of memory with accumulator |
| HLT | Hold the program |

### Implementation:

Input:

0001 = 30

0002 = 20

Output:

0003 = 50

PRACTICAL 4

AIM: Write an assembly language code in GNUsim8085 to implement Addition of two 16-bit Numbers.

Theory:

| Code | Meaning |
| --- | --- |
| LDA 2011 | Load value of memory location 0001 in Accumulator A |
| MOV B, A | Move data from memory to accumulator |
| STA 0003 | Store accumulator contents in memory |
| JNC L1 | Jump if no carry CY = 0 |
| INR C | Increment register or memory by 1 |
| ADC B | Add register or memory to accumulator with carry |
| ADD B | Add data of memory with accumulator |
| HLT | Hold the program |

IMPLEMENTATION:

INPUT:  	2011=82

2012=82

2014=80

2015=20

OUTPUT:    2013=164

2016=100

2017=2

PRACTICAL 5

AIM: Write an assembly language code in GNUsim8085 to implement Subtraction of two 8bit Numbers.

THEORY

| Code | Meaning |
| --- | --- |
| LDA 2052 | Load value of memory location 2011 in Accumulator A |
| MOV B, A | Move data from memory to accumulator |
| STA 2053 | Store accumulator contents in memory |
| SUB B | Subtract register or memory from accumulator |
| HLT | Hold the program |

Implementation:

Input:

2051 =  6

2052 =  5

Output:

2053 =  1

PRACTICAL 6

AIM: Write an assembly language code in GNUsim8085 to implement Subtraction of two 16bit Numbers.

THEORY

| Code | Meaning |
| --- | --- |
| LDA 2052 | Load value of memory location 2011 in Accumulator A |
| MOV B, A | Move data from memory to accumulator |
| STA 2053 | Store accumulator contents in memory |
| SUB B | Subtract register or memory from accumulator |
| HLT | Hold the program |
| MVI A,00 | 8-bit data is stored in the destination register or memory. (Move immediate 8-bit) |
| LHLD | Load H-L registers direct |

Implementation:

Input:

3000 =  18

3002 =  17

Output:

3003 =  1

3004 =  0

PRACTICAL 7

AIM: Write an assembly language code in GNUsim8085 to implement Multiplication of two 8bit Numbers.

THEORY

| Code | Meaning |
| --- | --- |
| LDA 2011 | Load value of memory location 2011 in Accumulator A |
| MOV B, A | Move data from memory to accumulator |
| STA 2013 | Store accumulator contents in memory |
| MVI A,00 | 8-bit data is stored in the destination register or memory. (Move immediate 8-bit) |
| DCR D | Decrement register or memory by 1. |
| JNZ LOOP | Jump if No Zero (Z = 0) to LOOP |
| ADD B | Add data of memory with accumulator |
| HLT | Hold the program |

IMPLEMENTATION:

INPUT:        2011 = 120

2012 = 2

OUTPUT:   2013 = 240

PRACTICAL 8

AIM: Write an assembly language code in GNUsim8085 to implement Division of two 8bit Numbers

THEORY

| Code | Meaning |
| --- | --- |
| LDA 2021 | Load value of memory location 2021 in Accumulator A |
| MOV B, A | Move data from memory to accumulator |
| MVI C,00 | 8-bit data is stored in the destination register or memory. (Move immediate 8-bit) |
| CMP B | Compare register or memory with accumulator |
| JC LOOP | Jump if Carry CY=1 |
| SUB B | Subtract register or memory from accumulator |
| INR C | Increment register or memory by 1 |
| JMP NEXT | Jump unconditionally (16 Bit Address) |
| HLT | Hold the program |

IMPLEMENTATION:

INPUT:      2020=8

2021=2

OUTPUT:  2023=0

2024=4

PRACTICAL 9

AIM: Write an assembly language code in GNUsim8085 to smallest numbers stored in memory

THEORY

| Code | Meaning |
| --- | --- |
| LDA 2001 | Load value of memory location 2001 in Accumulator A |
| MOV m, A | Move data from memory to accumulator |
| CMP B | Compare register or memory with accumulator |
| JC Lp1 | Jump if Carry CY=1 |
| JMP Lp2 | Jump unconditionally (16 Bit Address) |
| STA 2003 | Store accumulator direct(16 bit) |
| HLT | Hold the program |

IMPLEMENTATION:

INPUT:      2001=4

2002=3

2003=2

OUTPUT:  2024=3

PRACTICAL 10

AIM: Write an assembly language code in GNUsim8085 to larger numbers stored in memory

THEORY

| Code | Meaning |
| --- | --- |
| LDA 2001 | Load value of memory location 2001 in Accumulator A |
| MOV m, A | Move data from memory to accumulator |
| CMP B | Compare register or memory with accumulator |
| JC Lp1 | Jump if Carry CY=1 |
| JMP Lp2 | Jump unconditionally (16 Bit Address) |
| STA 2003 | Store accumulator direct(16 bit) |
| HLT | Hold the program |

IMPLEMENTATION:

INPUT:      2001=4

2002=3

OUTPUT:  2003=4

PRACTICAL 11

AIM: Write an assembly language code in GNUsim8085 to smallest numbers among series stored in memory

THEORY

| Code | Meaning |
| --- | --- |
| LXI H,2000 | Load register pair immediate |
| MOV B, M | Move data from memory to accumulator |
| INX H | register pair are incremented by 1 |
| DCR B | Decrement register or memory by 1 |
| CMP | Compare register or memory with accumulator |
| JC AHEAD | Jump if Carry CY=1 |
| JNZ LOOP | Jump if No Zero (Z = 0) to LOOP |
| STA 2009 | Store accumulator direct(16 bit) |
| HLT | Hold the program |

IMPLEMENTATION:

INPUT:      2000=5

2001=4

2002=3

2003=4

2004=1

2005=5

OUTPUT:  2009=1

PRACTICAL 12

AIM: Write an assembly language code in GNUsim8085 to Largest numbers among series stored in memory

THEORY

| Code | Meaning |
| --- | --- |
| LXI H,2001 | Load register pair immediate |
| MOV C, A | Move data from memory to accumulator |
| MVI B,00 | Move immediate 8-bit |
| JMP SKIP | Jump unconditionally (16 Bit Address) |
| DCR C | Decrement register or memory by 1 |
| CMP B | Compare register or memory with accumulator |
| JNC SET | Jump if no carry CY = 0 |
| JNZ BACK | Jump if No Zero (Z = 0) to BACK |
| STA 2009 | Store accumulator direct(16 bit) |
| HLT | Hold the program |

IMPLEMENTATION:

INPUT:      2001=4

2002=4

2003=3

2004=2

2005=1

OUTPUT:  2009=4

PRACTICAL 13

AIM: Write an assembly language code in GNUsim8085 to Addition of numbers of array stored in memory

THEORY

| Code | Meaning |
| --- | --- |
| LXI H,2001 | Load register pair immediate |
| MOV C, A | Move data from memory to accumulator |
| MVI B,00 | Move immediate 8-bit |
| ADD M | Add register or memory to accumulator |
| INX H | Increment register pair by 1 |
| DCR C | Decrement register or memory by 1 |
| JNZ BACK | Jump if No Zero (Z = 0) to BACK |
| STA 2009 | Store accumulator direct(16 bit) |
| HLT | Hold the program |

IMPLEMENTATION:

INPUT:      2001=4

2002=5

2003=6

2004=3

2005=1

OUTPUT:  2009=19

PRACTICAL 14

AIM: Write an assembly language code in GNUsim8085 to even numbers addition stored in memory

THEORY

| Code | Meaning |
| --- | --- |
| LXI H,2001 | Load register pair immediate |
| MOV C, A | Move data from memory to accumulator |
| MVI B,00 | Move immediate 8-bit |
| ANI 01 | Logical AND immediate with accumulator (CY is reset, AC is set) |
| JNZ SKIP | Jump if No Zero (Z = 0) to SKIP |
| ADD M | Add register or memory to accumulator |
| INX H | Increment register pair by 1 |
| DCR C | Decrement register or memory by 1 |
| STA 2009 | Store accumulator direct(16 bit) |
| HLT | Hold the program |

IMPLEMENTATION:

INPUT:      2001=5

2002=5

2003=6

2004=3

2005=1

2006=4

OUTPUT:  2009=10

PRACTICAL 15

AIM: Write an assembly language code in GNUsim8085 to Odd numbers addition stored in memory

THEORY

| Code | Meaning |
| --- | --- |
| LXI H,2001 | Load register pair immediate |
| MOV C, A | Move data from memory to accumulator |
| MVI B,00 | Move immediate 8-bit |
| ANI 01 | Logical AND immediate with accumulator(CY is reset, AC is set) |
| JNZ SKIP | Jump if No Zero (Z = 0) to SKIP |
| ADD M | Add register or memory to accumulator |
| INX H | Increment register pair by 1 |
| DCR C | Decrement register or memory by 1 |
| STA 2009 | Store accumulator direct(16 bit) |
| HLT | Hold the program |

IMPLEMENTATION:

INPUT:      2001=4

2002=5

2003=6

2004=3

2005=1

OUTPUT:  2009=10

PRACTICAL 16

AIM: Write an assembly language code in GNUsim8085 to arranging numbers in ascending order stored in memory

THEORY

| Code | Meaning |
| --- | --- |
| LXI H,2001 | Load register pair immediate |
| MOV C, M | Move data from memory to accumulator |
| DCR C | Decrement register or memory by 1 |
| INX H | Increment register pair by 1 |
| CMP M | Compare register or memory with accumulator |
| JC S1 | Jump if Carry CY=1 |
| DCX H | Decrement register pair by 1 |
| JNZ L | Jump if No Zero (Z = 0) to L |
| HLT | Hold the program |

IMPLEMENTATION:

INPUT:                                                              OUTPUT:

2001=4						2001=4

2002=1						2002=1

2003=3						2003=2

2004=2						2004=3

2005=4						2004=4

PRACTICAL 17

AIM: Write an assembly language code in GNUsim8085 to arranging numbers in descending order stored in memory

THEORY

| Code | Meaning |
| --- | --- |
| LXI H,2001 | Load register pair immediate |
| MOV C, M | Move data from memory to accumulator |
| DCR C | Decrement register or memory by 1 |
| INX H | Increment register pair by 1 |
| CMP M | Compare register or memory with accumulator |
| JNC S1 | Jump if no carry CY = 0 |
| DCX H | Decrement register pair by 1 |
| JNZ L | Jump if No Zero (Z = 0) to L |
| HLT | Hold the program |

IMPLEMENTATION:

INPUT:                                                              OUTPUT:

2001=5						2001=5

2002=1						2002=5

2003=5						2003=4

2004=2						2004=3

2005=3						2005=2

2006=4                                                                 2006=1

PRACTICAL 18

AIM: Write an assembly language code in GNUsim8085 to convert BCD to binary stored in memory

THEORY

| Code | Meaning |
| --- | --- |
| LDA 2000H | Load value of memory location 0001 in Accumulator A |
| MOV C, M | Move data from memory to accumulator |
| ANI 0fh | Logical AND immediate with accumulator(CY is reset, AC is set) |
| RRC | Rotate right accumulator(can rotate accumulator current content to right by 1-bit position) |
| MVI A,00 | 8-bit data is stored in the destination register or memory. (Move immediate 8-bit) |
| DCR B | Decrement register or memory by 1 |
| ADD D | Add register or memory to accumulator |
| JNZ S | Jump if No Zero (Z = 0) to S |
| HLT | Hold the program |

IMPLEMENTATION:

INPUT:

2000=64

OUTPUT:

2008=40